# Purpose

To describe the methods for the determination of total moisture present in coal samples.

# Scope

This procedure applies to the determination of total moisture in coal samples analyzed at SMJ Lab. It covers all coal samples submitted for routine testing, quality control, and reporting purposes.

# Normative Reference

## Indian Standard IS 436 (Part 1): 2024

Methods for Sampling of Coal and Coke.

## Indian Standard IS 1350 (Part 1): 2025

Methods of Test for Coal and Coke - Proximate Analysis.

# Definitions and Abbreviations

## Total Moisture

The coal which has been exposed to contact with water in the seam or in a washery, or coal and coke wetted by rain, may carry free or visible water. This water plus the moisture within the material, is referred to as total moisture.

# Safety Precautions

- Use appropriate personal protective equipment (PPE), such as heat-resistant gloves, when handling hot crucibles.

- Ensure proper ventilation when operating the muffle furnace to prevent exposure to fumes.

- Follow all laboratory safety protocols and guidelines.

# Apparatus

## Ventilated Air oven

Drying oven in which a constant and uniform temperature of 108 ±2°C can be maintained.

## Non corrodible or S.S Tray

Approximately 1000 sq.cm in area and 5 cm in depth.

## Weighing vessel

Shallow, approximately 40 cm² in area, made of silica, glass or stainless steel, with ground edges and fitted with ground cover.

# Reagents

None required for this test method.

# Sample Handling and Preparation

- Obtain a 1 kg coal sample for special moisture determination.

- Crush the coal so that it passes through a 12.5 mm square mesh sieve conforming to IS 460:1978.

- Ensure the prepared sample is representative and properly mixed.

- Place the sample in a sealed container immediately after preparation.

- Use a non-corrodible container to avoid contamination and moisture loss during storage and transport.

# Procedure

Determination of Total Moisture is carried out in two stages, i.e. Air Drying followed by Oven Drying.

## Air Drying

- Accurately weigh the sample and the container to the nearest 0.5 g (M₁).

- Transfer the sample from the container to a pre-weighed metal tray, spreading the material evenly on the tray.

- Dry the empty container in air and transfer any coal adhering to the sides onto the tray.

- Weigh the dry empty container (M₂).

- Calculate and record the mass of the sample taken for the test (M₁ - M₂).

- Allow the material on the tray to air-dry at atmospheric temperature in a well-ventilated, dust-free area.

- Periodically weigh the tray and sample during the air-drying process.

- Consider the air-drying complete when the change in mass during an hour is less than 0.1% of the sample mass.

- Record the final mass after air-drying (M₃).

## Oven Drying:

- Heat an empty weighing vessel in a drying oven at 108 ± 2°C.

- After heating, allow the empty vessel to cool for 20 minutes, and then weigh it (W₁).

- Crush the air-dried coal sample to pass through a 2.90mm IS sieve.

- Spread approximately 10 g of the crushed material uniformly in the pre-weighed vessel and weigh it (W₂).

- Place the uncovered vessel containing the sample in the drying oven and heat it at a temperature of 108 ±2°C.

- Continue heating until there is no further loss in mass, which typically takes 1.5 to 3 hours.

- After drying, replace the cover on the vessel and transfer it to a desiccator.

- Allow the covered vessel to cool in the desiccator for 20 minutes.

- After cooling, weigh the covered vessel containing the dried sample (W₃).

# Calculation:

The percentage loss in mass of the original sample in air-drying as:

| Air Dry Loss (ADL), X = | (M₂ – M₃) | x 100 |
| --- | --- | --- |
|  | (M₂ – M₁) |  |

The percentage loss in mass of the air-dried sample on oven drying as:

| Residual Moisture %, Y = | (W₂ – W₃) | x 100 |
| --- | --- | --- |
|  | (W₂ – W₁) |  |

Total Moisture % = X + Y (1 – X / 100)